CCTV IP Camera Simulator — Setup (Ubuntu + MediaMTX + FFmpeg)

Warning – Level 3 Document, please do not distribute

# Introduction

Simulate IP cameras on Ubuntu or WSL using MediaMTX (RTSP server) and FFmpeg (looped MP4 feed). Each RTSP path (/cam1, /cam2, …) acts like a separate camera you can add to exacqVision.

## What You Need

| Component | Details | Required? |
| --- | --- | --- |
| Ubuntu machine or VM | Ubuntu 20.04, 22.04, or 25. Physical PC or virtual machine (Hyper-V, VMware, VirtualBox, WSL) | Yes |
| Admin access | Ability to run sudo commands in a terminal | Yes |
| Video file (.mp4) | Any MP4 video to loop as the camera feed. If you have none, the simulator auto-generates a test pattern. | Recommended |
| Network connectivity | The NVR must be able to reach the Ubuntu machine on TCP port 8554. Same LAN is best for many cameras. | Yes |
| NVR / VMS | exacqVision or any other RTSP-capable system | Yes |

# Problem

You need repeatable camera streams for testing and demos without installing physical cameras.

# Cause

A simulator provides consistent RTSP streams and removes variables like lighting, motion, and camera availability.

# Solution

Run the two scripts (Step 1 + Step 2), then add RTSP URLs to exacqVision. For WSL, configure port forwarding for 8554.

- MediaMTX — the RTSP server that your NVR connects to

- FFmpeg — reads your MP4 file and loops it, publishing each camera as a separate RTSP path (/cam1, /cam2, ...)

You will then: choose a camera count, generate a MediaMTX config, start MediaMTX, and add the RTSP URLs to exacqVision.

ℹ NOTE Stream URL format: rtsp://<UBUNTU-IP>:8554/cam1 through rtsp://<UBUNTU-IP>:8554/cam180

# Quick Start (Copy / Paste)

### Step A — Go to the simulator folder

### Step B — Run Step 1 (installer)

### Step C — Copy your MP4 into videos/

### Step D — Run Step 2 (wizard)

### Step E — Verify locally

ffplay rtsp://127.0.0.1:8554/cam1 # press Q to quit

- When prompted, enter the number of cameras (for example: 5).

- sudo bash step-2-simulator-auto-start.sh

- WSL example: cp "/mnt/c/Users/YourUser/Downloads/video.mp4" ~/cctv-simulator/videos/camera.mp4

- cp /path/to/video.mp4 ~/cctv-simulator/videos/camera.mp4

- Tip: Select N for ONVIF unless you specifically need auto-discovery.

- sudo bash step-1-setup.sh

- cd ~/cctv-simulator

- Run these steps in order (copy/paste). Keep the simulator in a space-free Linux path: ~/cctv-simulator.

# Connectivity — Finding the Right IP Address

## Get the Ubuntu / WSL IP address

ip addr show ¦ grep 'inet ' ¦ grep -v 127.0.0.1

You will see one or more lines like:

inet 172.26.7.84/20 ... eth0

The IP is the number before the slash. Use this in exacqVision.

## Which IP to use — reference table

| Setup | IP to use in exacqVision |
| --- | --- |
| Ubuntu VM on same Windows host as exacqVision (Hyper-V / WSL) | Use the VM's 172.x.x.x address — direct local connection, fastest |
| Ubuntu on separate machine, same LAN as NVR | Use Ubuntu's 192.168.x.x or 10.x.x.x LAN address |
| Ubuntu remote, connected via Tailscale | Use Ubuntu's 100.x.x.x Tailscale address — limited bandwidth, avoid for 100+ cameras |

⚠ WARNING If your camera simulator runs in WSL on Windows, the NVR cannot connect to WSL directly. You must add a Windows port forward — see the WSL Port Forwarding section below.

## WSL Port Forwarding (Windows → WSL)

WSL runs behind Windows NAT. To make port 8554 reachable from the network, run these commands in PowerShell as Administrator on the Windows machine:

### Step 1 — Get the WSL IP

In the WSL terminal:

ip addr show eth0 ¦ grep inet

Note the IP — for example 172.26.7.84. Use this in the next step.

### Step 2 — Add port proxy

# Run in PowerShell as Administrator

netsh interface portproxy add v4tov4 listenport=8554 listenaddress=0.0.0.0 connectport=8554 connectaddress=172.26.7.84

### Step 3 — Open Windows firewall

# Run in PowerShell as Administrator

New-NetFirewallRule -DisplayName "RTSP-8554" -Direction Inbound -Protocol TCP -LocalPort 8554 -Action Allow

### Step 4 — Verify the proxy is active

netsh interface portproxy show all

You should see an entry: 0.0.0.0:8554 → 172.26.7.84:8554

### Step 5 — Test from the NVR machine

nc -zv 192.168.2.46 8554

You must see Connection succeeded before adding cameras in exacqVision.

⚠ WARNING WSL's IP address changes on every reboot. If cameras stop connecting after a reboot, delete the old proxy rule and re-add it with the new WSL IP: netsh interface portproxy delete v4tov4 listenport=8554 listenaddress=0.0.0.0 then re-run Step 2 with the new IP.

# Troubleshooting

## MediaMTX Does Not Start

Check the log immediately after attempting to start:

sudo cat /opt/cctv-simulator/logs/mediamtx.log

| Error in log | Fix |
| --- | --- |
| cannot unmarshal bool into ... authMethod | The authMethod line in mediamtx.yml is invalid. Regenerate the config using generate_config.py — do not add authMethod manually. |
| unknown field rtspTransports | This field was removed in MediaMTX v1.9.x. Regenerate the config — do not copy old config snippets. |
| unknown field rtspEncryption | Same as above — removed in v1.9.x. Regenerate the config. |
| bind: address already in use | Another process is using port 8554. Run: sudo pkill mediamtx then try again. |

## exacqVision Shows Connecting...

Work through these checks in order, stopping when you find the issue:

- Confirm MediaMTX is running:  ss -tlnp | grep 8554  — must show LISTEN

- Confirm FFmpeg is running (if using runOnInit):  pgrep -c ffmpeg  — must match camera count

- Test the stream locally on Ubuntu:  ffplay rtsp://127.0.0.1:8554/cam1  — must show video

- Test network connectivity from the NVR machine:  nc -zv <UBUNTU-IP> 8554  — must say Connection succeeded

- Check the URL in exacqVision — must be the full rtsp:// URL, not just the IP address

- Check for typos — a common mistake is mixing the Tailscale IP (100.x.x.x) with the local IP

## PA_OpenStream (-9986) Audio Error in exacqVision

This is a harmless warning. exacqVision is trying to play the camera's audio track through your PC speakers and cannot find an audio device. Video recording and live view are not affected.

To eliminate the warning, regenerate the config with audio disabled (the -an flag strips the audio track). The default generate_config.py already includes -an. If you are still getting the error, verify your config contains -an in the FFmpeg command:

grep 'ffmpeg' /opt/cctv-simulator/configs/mediamtx.yml ¦ head -1

The command should contain -an. If it does not, regenerate:

sudo python3 /opt/cctv-simulator/generate_config.py 180 /opt/cctv-simulator/videos/camera.mp4 /opt/cctv-simulator/configs

## Slow to Connect / Write Queue Full Warnings

If the MediaMTX log shows write queue is full, the network cannot sustain the current bitrate across all streams. Solutions:

- Use a local network connection instead of Tailscale

- Reduce the video bitrate by re-encoding to a lower resolution:

sudo ffmpeg -i /opt/cctv-simulator/videos/camera.mp4 \

-c:v libx264 -b:v 500k -maxrate 500k -bufsize 1000k \

-vf scale=640:360 -preset ultrafast -an \

/opt/cctv-simulator/videos/camera_low.mp4

- At 500 kbps, 180 cameras = ~90 Mbps total — manageable on a local network

## SyntaxError: invalid non-printable character U+00A0

You pasted Python code from a document or web page that contains invisible non-breaking space characters. These look like normal spaces but cause Python to reject the code.

Fix: never paste Python code blocks manually. Always use the generate_config.py script:

sudo python3 /opt/cctv-simulator/generate_config.py 180 /opt/cctv-simulator/videos/camera.mp4 /opt/cctv-simulator/configs

# Quick Reference — Essential Commands

## Day-to-day Operations

| Task | Command |
| --- | --- |
| Start MediaMTX | sudo mediamtx /opt/cctv-simulator/configs/mediamtx.yml & |
| Stop MediaMTX | sudo pkill mediamtx |
| Stop all FFmpeg streams | sudo pkill ffmpeg |
| Check if MediaMTX is running | ss -tlnp ¦ grep 8554 |
| Count active FFmpeg processes | pgrep -c ffmpeg |
| Watch FFmpeg count (wait for all to start) | watch -n2 'pgrep -c ffmpeg' |
| Test a stream locally | ffplay rtsp://127.0.0.1:8554/cam1 |
| View error log | sudo cat /opt/cctv-simulator/logs/mediamtx.log |
| Get Ubuntu / WSL IP address | ip addr show ¦ grep 'inet ' ¦ grep -v 127 |

## Changing Camera Count or Video

| Task | Command |
| --- | --- |
| Generate config for N cameras | sudo python3 /opt/cctv-simulator/generate_config.py N /opt/cctv-simulator/videos/camera.mp4 /opt/cctv-simulator/configs |
| Switch to runOnInit mode | sudo sed -i 's/runOnDemand:/runOnInit:/g; s/runOnDemandRestart/runOnInitRestart/g' /opt/cctv-simulator/configs/mediamtx.yml |
| Replace video file | sudo cp /path/to/new.mp4 /opt/cctv-simulator/videos/camera.mp4 |
| Replace video from Windows (WSL) | sudo cp /mnt/c/Users/YourUser/Downloads/new.mp4 /opt/cctv-simulator/videos/camera.mp4 |
| Generate test pattern video | ffmpeg -f lavfi -i testsrc2=size=1280x720:rate=25 -f lavfi -i sine=frequency=0 -c:v libx264 -preset ultrafast -t 60 -c:a aac /opt/cctv-simulator/videos/camera.mp4 |
| Re-encode to low bitrate (500k) | sudo ffmpeg -i /opt/cctv-simulator/videos/camera.mp4 -c:v libx264 -b:v 500k -vf scale=640:360 -preset ultrafast -an /opt/cctv-simulator/videos/camera_low.mp4 |

## File Locations

| File / Folder | Purpose |
| --- | --- |
| /opt/cctv-simulator/configs/mediamtx.yml | MediaMTX config — auto-generated by generate_config.py. Do not edit manually. |
| /opt/cctv-simulator/configs/cameras.conf | Your settings — edit camera count and video path here. |
| /opt/cctv-simulator/videos/camera.mp4 | Your video file — replace with real footage any time. |
| /opt/cctv-simulator/logs/mediamtx.log | Error and event log — check here first when troubleshooting. |
| /opt/cctv-simulator/generate_config.py | Regenerates mediamtx.yml. Run this after every camera count or video change. |